Technical Specification – 25 Ton Stainless Steel Mixing Tank

Client: SMS Demaj

General Requirements

- Tank nominal capacity: 25 Metric Tons (±5%)

- Product contact material: SS316

- Surface roughness of all product contact surfaces: Ra ≤ 0.8 μm

- Maximum overall equipment height: 6200 mm

Non-Destructive Testing (NDT)

The tank shall be subjected to Non-Destructive Testing as follows:

- Penetrant Testing (PT)

- Radiographic Testing (RT)

All weld seams in direct contact with the product shall be inspected by PT and RT methods.

A separate NDT report shall be included in the final documentation package.

Agitation System

- Agitator motor brand: SEW

- Mounting arrangement: Horizontal

- Motor power: 3 kW

- Output speed: 10–15 RPM

- Agitator shaft diameter: 60 mm minimum

The final motor and gearbox selection shall ensure a Service Factor (SF) greater than 1.

Process Connections
outer shell diameter :2930 mm
inner shell diameter : according to calculations

Product Inlet

- Diameter: 63.5 mm

Product Outlet

- Diameter: 50.8 mm

Instrumentation

The tank shall be equipped with:

Level Control

- High level switch

- Mid level switch

- Low level switch

Pressure Measurement

- Pressure Transmitter

- Manufacturer: WIKA

Temperature Measurement

- Thermometer

- Measuring range: 0–120 °C

Jacket System

- Jacket type: Dimple Jacket (Pillow Plate Jacket)

- Jacket height: 3900 mm

- Jacket thickness: 2 mm

The total jacket water volume shall be calculated and reported.

Tank Construction

Inner Shell

- Material: SS316

- Thickness: 3 mm

Inner Bottom Head

- Material: SS316

- Thickness: 4 mm

Top Head

- Material: SS316

- Thickness: 3 mm

Outer Cladding Shell

- Thickness: 2 mm

Insulation

- Material: Glass Wool

- Thickness: 50 mm

Support Structure

Legs

- Conical design

- Cone angle: 8.164°

Access and Safety

The tank shall be equipped with:

- Side manhole

- Access ladder

- Top guard

- Top working platform

Material Requirements

All components that are in direct contact with the product or product vapors shall be manufactured from SS316.

Any deviations from this requirement must be approved by the client prior to fabrication.

If the tank is equipped with a jacket, a hydrostatic test is required.

The jacket type is a Dimple Jacket (Pillow Plate Jacket).

The total water volume contained within the jacket shall be calculated and reported.

The sheet material required to cover the surfaces that require a jacket shall be calculated. The calculated quantity may be increased by up to 10% of the total calculated length.

If the load on each support exceeds 6 tons, adjustment bolts of grade 80 and Z8 should be used; otherwise, adjustment bolts of grade 40 and Z11 should be used.